TITLE

CEOP_Tsukuba_MRI_20090101_20090630.sfc

CONTACT

Sigenori Haginoya

Meteorological Research Institute

1-1, Nagamine Tsukuba Ibaraki 305-0052 JAPAN

E-mail: shaginoy@mri-jma.go.jp

DATE OF THIS DOCUMENT

24 September 2009

1. 0 DATASET OVERVIEW

  - Introduction

Intensive meteorological observations in the yard of Meteorological Research Institute, Tsukuba Japan, have been conducted since November of 2002 in order to provide long-term monitoring of the meteorological elements at Tsukuba area. This area, being located on the Kanto plains is typical suburbs area.  The surface of observation filed is covered by grass.  Grass is cut several times per year.

  - Time period covered by the data

Start: 1 January 2009, 00:00

End: 30 June 2009, 23:30

  - Temporal characteristics of the data

All parameters are recoded every 30 minutes intervals.

  - Physical location of the measurement

Latitude: 36° 03' 09" N

Longitude: 140° 07' 24" E

Elevation: 25.2 m a.s.l.

  - Data source

Original data is provided by MRI.

  - WWW address references

2.0 INSTRUMENTATION DESCRIPTION

2.1 Platform

The sensors are mounted on a 1.5m and a 6-m masts.

2.2 Description of the instrumentation

| Parameter | Model | Manifacturer |
| --- | --- | --- |
| Wind Speed | YG-5103 | Young (USA) |
| Wind Direction | YG-5103 | Young (USA) |
| Air Temperature | HMP45D | Vaisala (Finland) |
| Relative Humidity | HMP45D | Vaisala (Finland) |
| Radiation temperature | CML-303N | Climatec (Japan) |
| Downward Shortwave Radiation | CNR1 | Kipp&Zonen (The Netherlands) |
| Upward Shortwave Radiation | CNR1 | Kipp&Zonen (The Netherlands) |
| Downward Longwave Radiation | CNR1 | Kipp&Zonen (The Netherlands) |
| Upward Longwave Radiation | CNR1 | Kipp&Zonen (The Netherlands) |
| Net radiation | CNR1 | Kipp&Zonen (The Netherlands) |
| Precipitation | 34-T | Ota Keiki (Japan) |
| Pressure | PTB-200 | Vaisala (Norway) |

  - Instrumentation specification

| Parameter | Sensor Type | Height of sensor (m) | ## Accuracy | ## Resolution |
| --- | --- | --- | --- | --- |
| Wind Speed | vane | 7.50 | 0.1 m/s | 0.05 m/s |
| Wind Direction | Potentiometer | 7.50 | 1% | 0.1° |
| Air Temperature | Thermo-resistance | 1.50 | 0.1°C | 0.025°C |
| Relative Humidity | Capacitive Plate | 1.50 | 2.5% | 0.2% |
| Radiation temperature | thermopile | 0.70 | +/-(2+0.1%output range)°C: 0-200°C +/-(10%reading+2+0.1% output range) °C: below 0°C | 0.1°C |
| Downward Shortwave Radiation | thermopile | 1.60 | 10% (daily total) | 0.1W/m^2 |
| Upward Shortwave Radiation | thermopile | 1.60 | 10% (daily total) | 0.1W/m^2 |
| Downward Longwave Radiation | thermopile | 1.60 | 10% (daily total) | 0.1W/m^2 |
| Upward Longwave Radiation | thermopile | 1.60 | 10% (daily total) | 0.1W/m^2 |
| Net radiation | thermopile | 1.60 | 10% (daily total) | 0.1W/m^2 |
| Precipitation | Tipping Bucket | 0.80 | +/-0.5mm (<=20mm/hr) +/-3% (>20mm/h) | 0.5 mm |
| Pressure | Silicon capacitance | 5.30 | +/-0.2hPa (800-1050hPa) | 0.01hPa |

3.0 DATA COLLECTION AND PROCESSING

3.1 Description of data collection

Data are downloaded from the AWS every 30minutes. Then, data are sent to data server PC, where they are processed.

3.2 Description of derived parameters and processing techniques used

Temperature, relative humidity, radiation temperature and 4-components radiation are the previous 30 minutes average values.  Precipitation is accumulated on the previous 30 minutes.  Wind speed is scalar average and wind direction is vector average of unit vector over the previous 30 minutes (calculated by the datalogger by means of data recorded every 5 seconds).  Pressure data is used the Tateno Aerological Observatory apart from 400m north-east of the present site.  Original data is instantaneous hourly value.  30 minutes data is calculated the average of hourly data just before and after.

4.0 QUALITY CONTROL PROCEDURES

For all parameters, the data has been visually checked, looking for extremely low/high values and/or periods with constant values. Nocturnal shortwave radiation data has been checked for non-zero values; wind speed and direction for sensor freezing (in that case recorded wind speed is zero and wind direction is 360°); precipitation data was completely rejected due to sensor fault. The quality control flags follow the CEOP data flag definition document.

5.0 GAP FILLING PROCEDURES

No gap filling procedure was applied.

6.0 DATA REMARKS

6.1 PI's assessment of the data

6.1.1 Instruments problems

6.1.2 Quality issues

6.2 Missing data periods

7.0 REFERENCE REQUIREMENTS

Original data was collected and is provided within the framework of the CEOP Tsukuba Project, funded by...

8.0 REFERENCES