TITLE:

CEOP_Tsukuba_NIAES-MASE_20080701_20081231.sfc

CONTACT(S):

1) Akira Miyata

National Institute for Agro-Environmental Sciences

Tsukuba 305-8604, Japan

E-mail: amiyat@niaes.affrc.go.jp

2) Masayoshi Mano

National Institute for Agro-Environmental Sciences

Tsukuba 305-8604, Japan

E-mail: mmano@niaes.affrc.go.jp

DATE OF THIS DOCUMENT:

1 April 2009

1. 0 DATASET OVERVIEW:

  - Introduction:

Mase paddy flux site was established in 1999 to monitor greenhouse gas exchange between paddy fields and the atmosphere, and since then, Mase site is operated as one of the key study sites of AsiaFlux (http://www.asiaflux.net/). Details of the study site and instrumentation are given in some references (Saito et al., 2005; Miyata et al., 2005; Han et al., 2007; Saito et al., 2007).

  - Time period covered by the data:

Start: 1 July 2008, 00:00 (UTC)

End:  30 December 2008, 23:30 (UTC)

  - Physical location of the measurement:

Latitude: 36° 03' 14.3" N

Longitude: 140° 01' 36.9" E

Elevation: 11 m a.s.l.  Measurement site was not moved, but elevation data was updated.

Landscape: Agricultural fields (paddy fields)

Soil characteristics: Soil type is Eutric Fluvisols. The site is flooded most of rice growing season (from the beginning of May to mid-September).

  - Data source:

Original data.

  - WWW address references:

http://www.asiaflux.net/network/007MSE_1.html

http://ecomdb.niaes.affrc.go.jp/e_level_page.php?select_area=1045&select_site=1121&select_period=9999

2.0 INSTRUMENTATION DESCRIPTION:

2.1 Platform:

The sensors are mounted on a 6-m tall mast. Exceptions are a sonic anemometer and an open-path infrared gas analyzer, which are mounted at the top of another mast.

2.2 Description of the instrumentation:

| Parameter | Model | Manufacturer |
| --- | --- | --- |
| Station Pressure | PTA427 | Vaisala, Helsinki, Finland |
| Air Temperature | HMP45A with home-made aspirator | Vaisala, Helsinki, Finland |
| Dew Point Temperature | Calculated | - |
| Relative Humidity | HMP45A with home-made aspirator | Vaisala, Helsinki, Finland |
| Specific Humidity | Calculated |  |
| Wind Speed | AF750 | Makino Applied Instruments, Tokyo, Japan |
| Wind Direction | VF016 | Makino Applied Instruments, Tokyo, Japan |
| U wind component | Calculated. | - |
| V wind component | Calculated | - |
| Precipitation | TE525MM | Texas Electronics, Dallas, TX, USA |
| Snow Depth | - | - |
| Incoming Shortwave | CNR1 (CM3) | Kipp&Zonen, Delft, the Netherlands |
| Outgoing Shortwave | CNR1 (CM3) | Kipp&Zonen, Delft, the Netherlands |
| Incoming Longwave | CNR1 (CG3) | Kipp&Zonen, Delft, the Netherlands |
| Outgoing Longwave | CNR1 (CG3) | Kipp&Zonen, Delft, the Netherlands |
| Net Radiation | CNR1 | Kipp&Zonen, Delft, the Netherlands |
| Skin Temperature | IRR-P | Campbell, Logan, UT, USA |
| Incoming PAR | LI190 | LICOR, Lincoln, NE, USA |
| Outgoing PAR | LI190 | LICOR, Lincoln, NE, USA |

  - Instrumentation specification:

| Parameter | Sensor Type | Height of sensor (m) | ## Accuracy | ## Resolution |
| --- | --- | --- | --- | --- |
| Station Pressure | Silicon capacitive sensor | 1.0 | 0.5 hPa | - |
| Air Temperature | Platinum resistance thermometer | 3.75 | 0.2 | - |
| Dew Point Temperature | - | - | - | - |
| Relative Humidity | Humicap (capacitive thin-film polymer sensor) | 3.75 | 2% RH (0-90% RH) 3% RH (90-100% RH) | - |
| Specific Humidity | - | - | - | - |
| Wind Speed | Cup anemometer | 3.75 | - | 0.2 m/s (starting velocity) |
| Wind Direction | Wind vane | 4.45 | - | - |
| U wind component | - | - | - | - |
| V wind component | - | - | - | - |
| Precipitation | Tipping-bucket rain gauge | 1.5 | 1% | 0.1 mm |
| Snow Depth | - | - | - | - |
| Incoming Shortwave | Pyranometer | 2.5 | 10% | - |
| Outgoing Shortwave | Pyranometer | 2.5 | 10% | - |
| Incoming Longwave | Pyrgeometer | 2.5 | 10% | - |
| Outgoing Longwave | Pyrgeometer | 2.5 | 10% | - |
| Net Radiation | Combination of a pair of pyranometers and pyrgeometers | 2.5 | 10% | - |
| Skin Temperature | Infrared thermometer | - | 0.2 K |  |
| Incoming PAR | Silicon photodiode | 2.5 | 5% | - |
| Outgoing PAR | Silicon photodiode | 2.5 | 5% | - |

3.0 DATA COLLECTION AND PROCESSING:

3.1 Description of data collection:

Data are retrieved weekly.

3.2 Description of derived parameters and processing techniques used:

1) Except for precipitation and wind components, instantaneous values were sampled every 5 seconds, and their 30-minute averages were calculated on line and stored.

2) For some key parameters such as radiation, standard deviations were also calculated and stored. For precipitation, the total of previous 30-minute period was recorded.

3) 30-minute averages of dew point temperature and specific humidity can be calculated from 30-minute averages of air temperature and relative humidity.

4) U and V wind components were calculated from 30-minute averages of wind speed and wind direction. Caution has to be taken that the 30-minute average of wind direction is the scalar average of instantaneous wind direction (0-540 degree)

4.0 QUALITY CONTROL PROCEDURES:

At this stage of data processing, only apparently erroneous data were removed. Further quality control of the data will be done later.

5.0 GAP FILLING PROCEDURES:

At this stage of data processing, no gap filling procedure was applied. Gap filling of key parameters will be done later.

6.0 DATA REMARKS:

6.1 PI's assessment of the data:

6.1.1 Instruments problems

Infrared thermometer to measure skin temperature malfunctioned until 2008/5/16 8:30:00 UTC.

6.1.2 Quality issues

1) Wind speed may contain errors because the cup anemometer had not been calibrated for more than two years. Intercomparison with sonic anemometer will be done later.

2) Sensors for air temperature and relative humidity were inter-calibrated with another sensor of the same model to remove influence of instrumental errors on measurement of vertical gradient, but their absolute values may contain small offset errors.

3) Since radiation sensors have small offset errors, nighttime values of short-wave radiation and PAR were not exactly zero. PAR values may contain errors due to long-term changes of the sensitivities. Calibration of the PAR sensors will be made after the 2008 growing season.

6.2 Missing data periods:

1) from 2008/01/01 00:00 to 2008/03/03 04:30 (UTC) (Skin Temperature)

2) from 2008/03/04 05:00 to 2008/03/07 08:00  (UTC) (All data)

3) from2008/03/07 08:30 to 2008/04/03 00:00 (UTC) (Wind Speed, Wind Direction, Skin Temperature)

4) from 2008/04/03 00:30 to 2008/05/05 05:00 (UTC) (All data)

5) 2008/05/07 03:30  (UTC) (Skin Temperature)

6) from 2008/05/12 07:00 to 2008/05/12 08:00 (UTC) (All data)

7) from 2008/05/12 09:00 to 2008/05/15 00:30 (UTC) (Skin Temperature)

8) from 2008/05/15 07:30 to 2008/05/16 08:30 (UTC) (Skin Temperature)

9) from 2008/05/15 08:00 (UTC) (Outgoing PAR)

10) from 2008/05/16 08:00 to 2008/05/16 08:30 (UTC) (Outgoing PAR)

11) from 2008/06/20 15:30 to 2008/06/20 21:30 (UTC) (Relative Humidity)

12) from 2008/06/28 19:00 to 2008/06/29 21:30 (UTC) (Relative Humidity)

6.3 Data intercomparisons:

7.0 REFERENCE REQUIREMENTS:

Original data were collected in the framework of Research Project for Global Warming Monitoring by NIAES. The project is funded by Ministry of Agriculture, Forestry and Fisheries, Ministry of Environment and NIAES.

8.0 REFERENCES

Saito, M, A. Miyata, H. Nagai, and T. Yamada, Seasonal variation of carbon dioxide exchange in rice paddy field in Japan. Agric. Forest Meteorol. 135, 93-109, 2005.

Miyata, A., T. Iwata, H. Nagai, T. Yamada, H. Yoshikoshi, M. Mano, K. Ono, G. H. Han, Y. Harazono, E. Ohtaki, Md. A. Baten, S. Inohara, T. Takimoto, and M. Saito, Seasonal variation of carbon dioxide and methane fluxes at single cropping paddy fields in central and western Japan, Phyton, 45(4), 89-97, 2005.

Saito, M., J. Asanuma, A. Miyata, Dual-scale transport of sensible heat and water vapor over a short canopy under unstable conditions. Water Resources Research, 43, W05413, doi:10.1029/2006WR005136, 2007.

Han, G.H., H. Yoshikoshi, H. Nagai, T. Yamada, K. Ono, M. Mano, A. Miyata, Isotopic disequilibrium between carbon assimilated and respired in a rice paddy as influenced by methanogenesis from CO2. Journal of Geophysical Research, 112, G02016, doi:10.1029/2006JG000219, 2007.