TITLE

CEOP_Tsukuba_NIED_20070701_20071231.sfc

CONTACT

Takeshi Maesaka

Storm, Flood, and Landslide Research Department

National Research Institute for Earth Science and Disaster Prevention

3-1 Tennodai Tsukuba Ibaraki 305-0006 Japan

E-mail: maesaka@bosai.go.jp

DATE OF THIS DOCUMENT

31 January 2008

1. 0 DATASET OVERVIEW

  - Introduction

Hydrometeorological observations at NIED, Tsukuba have been intermittently conducted since the late 90's to verify the monitoring systems which were installed in the Asian monsoon region in Southeast Asia. Nowadays, NIED has an ongoing research project to establish a multi-parameter radar network and associated ground observations (precipitation, temperature, wind velocity, soil moisture, etc.) in the Kanto plain for the short-term prediction of severe storms, floods and landslides. To verify the monitoring system of the ground observation and to compare with the additional hydrometeorological parameters which are not included in the ground observation in the project, the monitoring has been conducted in the NIED experimental field.

  - Time period covered by the data

Start: 01 July 2007, 00:00

End: 31 December 2007, 23:30

  - Temporal characteristics of the data

All parameters are recoded every 1 minute.

  - Physical location of the measurement

Latitude: 36° 07' 33.0" N

Longitude: 140° 05' 23.9" E

Elevation: 24 m a.s.l.

  - Data source

Original data provided by the NIED ground observation team.

  - WWW address references

2.0 INSTRUMENTATION DESCRIPTION

2.1 Platform

The sensors are the WEATHER MULTI-SENSOR.

2.2 Description of the instrumentation

| Parameter | Model | Manifacturer |
| --- | --- | --- |
| Air temperature | WXT510 (TERMOCAP) | Vaisala |
| Relative humidity | WXT510 (HUMICAP) | Vaisala |
| Wind Speed | WXT510 (WINDCAP) | Vaisala |
| Wind Direction | WXT510 (WINDCAP) | Vaisala |
| Air pressure | WXT510 (BAROCAP) | Vaisala |
| Rainfall Intensity | WXT510 (RAINCAP) | Vaisala |

  - Instrumentation specification

| Parameter | Sensor Type | Height of sensor (m) | Accuracy | Resolution |
| --- | --- | --- | --- | --- |
| Air temperature | Capacitive ceramic | 1.5 m AGL | 0.3 °C | 0.1 °C |
| Relative humidity | Capacitive thin film polymer | 1.5 m AGL | 3 % (0 - 90 %) 5 % (90 -100%) | 0.1 % |
| Wind Speed | Ultrasonic | 1.5 m AGL | 0.3 m/s or 3% (0 - 35 m/s) 5 m/s (36 - 60 m/s) | 0.1 m/s |
| Wind Direction | Ultrasonic | 1.5 m AGL | 3 ° | 1 ° |
| Air pressure | Capacitive silicon | 1.5 m AGL | 0.5 hPa (0 - 30 °C) 1 hPa (-52 – 60 °C) | 0.1 hPa |
| Rainfall Intensity | Piezoelectrical sensor | 1.5 m AGL | better than 5 % for daily accumulation | 0.1 mm/hour |

3.0 DATA COLLECTION AND PROCESSING

3.1 Description of data collection

Data are downloaded from the WEATHER MULTI-SENSOR online.

3.2 Description of derived parameters and processing techniques used

See manufacture’s web page (http://www.vaisala.com/).

4.0 QUALITY CONTROL PROCEDURES

For all parameters, the data has been visually checked, looking for extremely low/high values and/or periods with constant values. Original data consists of 1 minute interval, with 4 Hz sampling for winds and pre-one-minute accumulated amount for precipitation.  30-minuets data are constructed by averaging or accumulating of their values with flag of I. The quality control flags follow the CEOP data flag definition document.

5.0 GAP FILLING PROCEDURES

No gap filling procedure was applied.

6.0 DATA REMARKS

6.1 PI's assessment of the data

6.1.1 Instruments problems

6.1.2 Quality issues

6.2 Missing data periods

There are frequent and irregular missing in all factors.

7.0 REFERENCE REQUIREMENTS

Original data were collected and are provided under funding support of the Research Project of the National Research Institute for Earth Science and Disaster Prevention.

8.0 REFERENCES