TITLE

CEOP_Tsukuba_NIFTS_2009101_2009630.sfc

CONTACT

Hiroyoshi Sugiura

National Institute of Fruit Tree Science (NIFTS)

2-1, Fujimoto, Tsukuba, Ibaraki 305-8605 JAPAN

E-mail: hsugiura@affrc.go.jp

DATE OF THIS DOCUMENT

21 June 2010

1. 0 DATASET OVERVIEW

  - Introduction

NIFTS is located in suburban in Tsukuba. Meteorological observation site is close to orchards. The observation has been conducted since 1978 in order to compare meteorological elements and fruit tree growth.

  - Time period covered by the data

Start: 1 January 2009 00:00

End: 30 June 2009 23:30

  - Temporal characteristics of the data

All parameters are recoded every 30 minutes intervals.

  - Physical location of the measurement

Latitude: 36° 02' 54" N

Longitude: 140° 05' 58" E

Elevation: 39.6 m a.s.l.

  - Data source

Original data is provided by NIFTS.

  - WWW address references

http://www.fruit.affrc.go.jp/index-e.html

2.0 INSTRUMENTATION DESCRIPTION

2.1 Platform

The thermometer, humidity-meter, wind vane and pyranometer are mounted on the tower. The precipitation gauge is set on the ground.

2.2 Description of the instrumentation

| Parameter | Model | Manifacturer |
| --- | --- | --- |
| Air Temperature | HMP-45D | Vaisala (Norway) |
| Precipitation (from 1 Jan.-4 Mar.) | B-011-1 | Yokokawa (Japan) |
| Precipitation (from 5 Mar.-) | 34-T | Ohta (Japan) |
| Relative Humidity | HMP-45D | Vaisala (Norway) |
| Wind Speed | 05103 | R.M.Young (USA) |
| Wind Direction | 05103 | R.M.Young (USA) |
| Downward Shortwave Radiation | MS-43F | Eko (Japan) |

  - Instrumentation specification

| Parameter | Sensor Type | Height of sensor (m) | ## Accuracy | ## Resolution |
| --- | --- | --- | --- | --- |
| Air Temperature | Pt100 | 1.5 m above the ground | 0.2°C | 0.1°C |
| Precipitation | Tipping-buket/heater | 0.5m above the ground | 0.5mm | 0.5 mm |
| Relative Humidity | High polymar firm | 1.5 m above the ground | 2 % (0-90%) 3 % (above 90%) | 0.1% |
| Wind Speed | Wind vane | 11 m above the ground | 0.3 m/s | 0.1 m/s |
| Wind Direction | Wind vane | 11 m above the ground | 3 deg | 1 deg |
| Downward Shortwave Radiation | Termopile | 8 m above the ground | 0.1%/℃ | 0.1 W/m2 |

3.0 DATA COLLECTION AND PROCESSING

3.1 Description of data collection

Data are downloaded from the system every 10 minutes, then sent to data server PC and converted to the meteorological elements.

3.2 Description of derived parameters and processing techniques used

30-minute interval data are converted from instantaneous 10-minute interval data by CEOP-Tsukuba data center (University of Tsukuba) as follows.  Air temperature, relative humidity, wind direction and speed, incoming shortwave, soil temperature are averaged (precipitation is accumulated) during a 30-minuets interval before the time stamp.   Wind data are the vector average of (u, v) components observed in 10-minuets interval.  Dew point data is calculated using temperature and relative humidity data, based on equations in http://www.eol.ucar.edu/projects/ceop/dm/documents/refdata_report/eqns.html.

4.0 QUALITY CONTROL PROCEDURES

No specific quality control. If there is a missing data in the original 10 minutes interval data, the 30 minuets interval data is treated as NODATA (-999.9).

The quality control flags follow the CEOP data flag definition document.

5.0 GAP FILLING PROCEDURES

No gap filling procedure was applied.

6.0 DATA REMARKS

6.1 PI's assessment of the data

6.1.1 Instruments problems

6.1.2 Quality issues

6.2 Missing data periods

If there is at least one missing datum existed in the original data, 30-minute interval data with CEOP format are treated as missing (-999.99).  Periods with the missing data are listed below.

Missing except for precipitation

5 March, 0:30-6:00 UTC

Missing for precipitation

5 March, 0:30-6:30 UTC

7.0 REFERENCE REQUIREMENTS

8.0 REFERENCES