TITLE

CEOP_Tsukuba_TERC-UT_20070701_20071231.sfc

CONTACT

Jun Asanuma

Terrestrial Environment Research Center, University of Tsukuba

Tenno-dai 1-1-1

Tsukuba, Ibaraki 305-8572 Japan

E-mail: asanuma@suiri.tsukuba.ac.jp

DATE OF THIS DOCUMENT

10 Jul. 2008

1. 0 DATASET OVERVIEW

  - Introduction

Terrestrial Environment Research Center (TERC) started its long-term measurement of the surface heat and water balance at its surface heat and water balance field in 1981, and is continuing the observation. The field is circle in shape and has a 30-meter tower at its center. On the tower, profiles of air temperature, humidity and turbulence fluxes are measured and while, on the ground of the field, various measurements relevant to the surface heat and water budget are also being made. These include radiation components, ground water levels, soil temperature and soil heat flux. The measured data averaged over various time scale (30min., 1hour, 1day) are available on the web page (http://www.suiri.tsukuba.ac.jp). Some of the can be dated back to 1980’s. Among the measurements, basic meteorological and hydrological parameters are provided for CEOP2.

  - Time period covered by the data

Start: 1 July 2007, 00:00 UTC

End: 31 December 2007, 23:30 UTC

  - Temporal characteristics of the data

Original data, from which the data submitted to CEOP2, is averaged for 30 minutes.

  - Physical location of the measurement

Latitude:	36°06′35" N

Longtitude:	140°06′00" E

Altitude:	27.0 m a.s.l.

  - Data source

Original data provided by the Terrestrial Environment Research Center, University of Tsukuba. Dew-point temperature, specific humidity are calculated using the original data.

  - WWW address references

http://www.suiri.tsukuba.ac.jp/

2.0 INSTRUMENTATION DESCRIPTION

2.1 Platform

On the tower, thermometers and humidity-meters are mounted at 1.6 m above the ground. Also, a sonic anemometer-thermometers are mounted at 1.6 m. A pyranometer and a tipping-bucket raingauge is placed on the ground.

2.2 Description of the instrumentation

| Parameter | Model | Manifacturer |
| --- | --- | --- |
| Wind Speed | DA-600 | Kaijo Inc |
| Air temperature | HMP-45D | Vaisala |
| Relative Humidity | HMP-45D | Vaisala |
| Air pressure | PTB-210 | Vaisala |
| Shortwave radiation | MS-402F | Eiko Inc. |
| Precipitation | WB0013-05 | Yokokawa Inc. |

  - Instrumentation specification

| Parameter | Sensor Type | Height of sensor (m) | ## Accuracy | ## Resolution |
| --- | --- | --- | --- | --- |
| Wind Speed | Sonic Anemometer-thermometer | 1.6 | 0.005m/s | 0.1m/s |
| Air temperature |  | 1.6 | 0.2 deg | 0.1 deg |
| Relative Humidity | High polymar firm | 1.6 | 2 % (0-90%) 3 % (above 90%) | 1% |
| Air pressure | silicon capacitive sensor | 1.5 | 0.30 hPa | 0.1hPa |
| Shortwave radiation | Thermopile | 1.5 | 8 W/m2 | 1W/m2 |
| Precipitation | Tipping bucket | 0. | 0.5mm | 0.5mm |

3.0 DATA COLLECTION AND PROCESSING

3.1 Description of data collection

Measurements of sonic anemometers are sampled at 10Hz, and those of the other sensors at every 10 seconds, and they are averaged over 30 minutes in the loggers. Averaged date stored in the these loggers are retrieved simultaneously through the local network and they are stored in the data server of TERC. These data are subject to the post-processed for CEOP2

3.2 Description of derived parameters and processing techniques used

Dew point temperature, specific humidity were computed from the relative humidity, the temperature, and air pressure.

4.0 QUALITY CONTROL PROCEDURES

Measurements during the maintenance of the instruments as well as other possible causes such as electric shutdown are marked as “m” (missing), and the data value is replace with the value -999.99. The measurements that are out of the ordinary range is marked as “B” (bad). When external influence, such as maintenance works, are possible the data is marked as “D”(dubious). Otherwise, the data is marked as “G” (good).

5.0 GAP FILLING PROCEDURES

No gap filling procedure was applied.

6.0 DATA REMARKS

6.1 PI's assessment of the data

6.1.1 Instruments problems

6.1.2 Quality issues

Data quality is almost good.

6.2 Missing data periods

Regular electric shutdown occurred between Oct 13 and  Oct 15, and no data were recorded during this time.

Grass mowning works on July 4, Oct 18 and  Oct 22.

Regular maintenance was made on Oct 15, Nov 5, Nov 15, Nov 21, Nov. 26, Dec 5, and Dec 14.

Quality flags are marked “D”, “M” or “G” depending on the possible influence on the measured values.

7.0 REFERENCE REQUIREMENTS

8.0 REFERENCES